Perfect 👍 — you’ve uploaded CA_Lecture2.pptx.pdf, which covers Computer Architecture basics, ISA (Instruction Set Architecture), addressing modes, parallelism, Flynn’s taxonomy, power/performance concepts, and design hierarchy.

Below is a fully written, simple yet detailed version of your lecture notes — explained in easy English (not just bullet points), with examples and real-life analogies so you can easily understand each concept.

# 🧠 Computer Architecture – Lecture 2 (Detailed Notes in Simple English)

## 🌟 1. Introduction

Every computer has two main design views:

- Computer Architecture

- Computer Organization

These two are often confused but mean different things.

### 💡 Computer Architecture

- It is the functional design of a computer — it describes what the computer does.

- It defines the behavior, not the physical structure.

- It focuses on the Instruction Set Architecture (ISA), data formats, addressing modes, and the functions each part must perform.

- In simple words, it is like the blueprint of how a computer should behave.

📘 Example:
Architecture decides “the CPU should support 32-bit addition and have 16 registers.”

### ⚙️ Computer Organization

- It describes how the computer actually works — how each part (ALU, registers, buses) is connected and operated to perform the architectural design.

- It’s about hardware structure and control signals.

- It comes after architecture — it shows “how to build what architecture describes.”

📘 Example:
Organization decides “we will use two ALUs, one adder circuit, and 16 flip-flops to make 16 registers.”

### 🔁 In Summary:

| Aspect | Computer Architecture | Computer Organization |
| --- | --- | --- |
| Definition | What the computer does | How the computer does it |
| Focus | Functional behavior (ISA, addressing, data types) | Physical implementation (hardware & control) |
| Example | "It can add two 32-bit numbers" | "It uses ripple carry adder circuits" |
| Level | Logical / Conceptual | Physical / Structural |

## 🧩 2. Levels of Abstraction in Computer Systems

A computer can be understood at several levels, from very high (software) to very low (electronic circuits).

### 🔹 1. Application Level

- At this level, we see software (like Chrome, Word, or games) running on the computer.

- The hardware details are hidden.

- Programmers just use functions like “add two numbers” — not caring about bits or registers.

### 🔹 2. Instruction Set Architecture (ISA)

- The interface between software and hardware.

- Defines the set of instructions the CPU can execute (like ADD, SUB, MOV).

- Also defines registers, addressing modes, and data types.

📘 Example:
Instruction: ADD R1, R2, R3
→ Adds R2 + R3 and stores the result in R1.

### 🔹 3. Microarchitecture

- Defines how the CPU executes those instructions.

- Includes internal components: ALU, control unit, registers, and cache memory.

- Determines the speed and performance of the processor.

📘 Example:
Two computers can have the same ISA (say ARM), but one is faster because its microarchitecture is pipelined or multi-core.

### 🔹 4. Circuit Level

- Describes the hardware made from logic gates, transistors, and flip-flops.

- This is the physical layer that makes all the higher levels work.

📘 Example:
The ALU is made using AND, OR, XOR gates; memory is built from latches and flip-flops.

## ⚙️ 3. Basic Concepts to Manage Complexity

When designing computers, engineers follow 3 key principles:

- Hierarchy:
Break the system into smaller parts — CPU, memory, input/output — then break each part further.
🧩 Like building a car: body, engine, wheels — each designed separately.

- Modularity:
Each module should have a clear function and interface (how it connects to others).
🧱 Example: ALU module only performs arithmetic; it connects to registers via the bus.

- Regularity:
Keep design uniform. Reuse similar modules to reduce complexity.
🧰 Example: All registers are 8-bit wide; all adders use the same circuit.

## 🧱 4. Digital Design and Abstraction Levels

Computers are designed in layers of abstraction, from high-level behavior to physical layout.

| Level | Description | Example |
| --- | --- | --- |
| System Design (PMS level) | Describes processors, memories, and interconnections | CPU connected to RAM via data bus |
| Instruction Set Level | Defines functions of instructions | “ADD” performs addition |
| Register Transfer Level (RTL) | Describes how data moves between registers | R1 ← R2 + R3 |
| Logic Level | Describes gates and flip-flops | AND, OR, NOT circuits |
| Circuit Level | Transistors and wires | CMOS design |

🧠 High-level synthesis → logic synthesis → layout synthesis (the path from abstract design to hardware).

## ⚡ 5. Types of Parallelism

Modern computers perform many tasks at once. This is called parallelism — doing multiple things simultaneously.

### 1️⃣ Data Level Parallelism (DLP)

- Same operation on multiple data items.
📘 Example: Add 1000 pairs of numbers in parallel.

### 2️⃣ Task Level Parallelism (TLP)

- Different tasks run at the same time.
📘 Example: One core runs Chrome, another runs Spotify.

### 3️⃣ Instruction Level Parallelism (ILP)

- Multiple instructions of one program executed together (via pipelining).
📘 Example: While instruction 1 executes, instruction 2 is fetched.

### 4️⃣ Thread Level Parallelism (TLP in hardware)

- Executes multiple threads (lightweight processes) in parallel.

### 5️⃣ Request Level Parallelism

- Independent requests handled at once (used in web servers).

## 🧮 6. Flynn’s Taxonomy (1966)

Michael Flynn classified computers by the number of instructions and data streams they process simultaneously.

| Type | Full Form | Meaning | Example |
| --- | --- | --- | --- |
| SISD | Single Instruction Single Data | One instruction on one data at a time (normal single CPU) | Old PCs |
| SIMD | Single Instruction Multiple Data | One instruction acts on many data at once | GPUs, vector processors |
| MISD | Multiple Instruction Single Data | Rare, used for redundancy or fault tolerance | Space systems |
| MIMD | Multiple Instruction Multiple Data | Many processors execute different instructions on different data | Modern multi-core CPUs |

📘 Example:
A modern 8-core CPU running 8 threads → MIMD.

## 🏗️ 7. Classical Processor Architectures

### 🧠 1. Von Neumann (Princeton) Architecture

- One shared memory for both data and instructions.

- The CPU fetches either instructions or data one at a time — not both.

- Simpler but slower.

📘 Example:
Traditional PCs follow this model.

### ⚙️ 2. Harvard Architecture

- Has separate memories for data and instructions.

- CPU can fetch data and instruction simultaneously, improving speed.
📘 Example:
Microcontrollers and DSP processors use this design.

## 💻 8. CPU Types

### 🔹 1. CISC (Complex Instruction Set Computer)

- Has many complex instructions.

- Each instruction can do multiple things.

- Instructions have variable lengths.

- Harder to pipeline.
📘 Example: Intel x86 family.

### 🔹 2. RISC (Reduced Instruction Set Computer)

- Few, simple, fast instructions.

- All instructions same size (easy to decode).

- Uses load/store mechanism — only load/store access memory.

- Ideal for pipelining.
📘 Examples: ARM, MIPS.

### 🔹 3. Other Architectures

- DSP – Digital Signal Processors (for audio/video).

- Embedded / SoC – System on Chip for small devices.

- FPGA – Reconfigurable hardware for flexible designs.

## 🧾 9. Instruction Set Architecture (ISA)

ISA defines the interface between software and hardware.
It includes:

- Registers – general, special, or kernel registers.

- Data types – integers, floating points, characters.

- Instruction set – all possible operations CPU can perform.

- Addressing modes – ways to access data.

- Flow control – jumps, loops, procedure calls.

- I/O and Interrupt handling.

📘 Examples of ISAs: ARM, MIPS, SPARC, Intel x86, RISC-V.

## 🧮 10. Addressing Modes

Addressing mode = how CPU finds the data it needs.

| Addressing Mode | Example | Meaning |
| --- | --- | --- |
| Register | ADD R4, R3 | Add contents of R3 to R4 |
| Immediate | ADD R4, #3 | Add constant 3 to R4 |
| Displacement | ADD R4, 100(R1) | Add data at (R1+100) to R4 |
| Register Indirect | ADD R4, (R1) | Add data at memory address stored in R1 |
| Indexed | ADD R4, (R1+R2) | Add data from address (R1+R2) |
| Direct | ADD R4, (1001) | Add data from memory[1001] |
| Auto-increment | ADD R4, (R3)+ | Use data from R3, then increase R3 |
| Auto-decrement | ADD R4, -(R3) | Decrease R3, then use that address |
| Scaled | ADD R4, 100(R2)[R3] | Add scaled address data to R4 |

📘 Example:
ADD R2, #5 → Adds immediate 5 to the value in R2.

## 🧠 11. Endianness

It defines how multi-byte data is stored in memory.

| Type | Description | Example (number = 12345678h) |
| --- | --- | --- |
| Little Endian | Least Significant Byte stored at lowest address | Memory: 78 56 34 12 |
| Big Endian | Most Significant Byte stored first | Memory: 12 34 56 78 |

📘 Example:
Intel uses Little Endian, older Motorola CPUs used Big Endian.

## 🧩 12. Instruction Operations and Classes

### 🔹 Operation Types:

- Arithmetic/Logic: ADD, SUB, AND, OR, MULTIPLY

- Data Transfer: MOV, LOAD, STORE

- Control: JUMP, CALL, RETURN

- System: OS instructions

- String & Graphics: Special purpose

## 🧮 13. Instruction Formats

- Fixed-length → All instructions have same size (e.g., 32-bit ARM).

- Variable-length → Each instruction can have different size (e.g., Intel x86).

- Hybrid → Mix of both.

📘 Example:

- ARM: 32-bit fixed

- x86: 1–18 bytes variable

- MIPS16/Thumb: hybrid (16/32-bit mix)

## 🧠 14. Basic ISA Classes (by number of addresses)

| Type | Description | Example |
| --- | --- | --- |
| 0-address (Stack) | Operands are on top of stack | PUSH A, PUSH B, ADD, POP C |
| 1-address (Accumulator) | One operand is the accumulator | LOAD A, ADD B, STORE C |
| 2-address (Register-Memory) | Two operands, result stored in one | ADD A, B |
| 3-address (Load/Store) | All operands explicit | ADD R1, R2, R3 (R1 = R2 + R3) |

📘 Example (for e = ab + cd):

- Stack: use PUSH/POP

- Accumulator: LOAD, MUL, ADD, STORE

- 3-address: use separate registers for each variable.

## ⚡ 15. Moore’s Law

- Proposed by Gordon Moore (1965).

- States: “The number of transistors on a chip doubles roughly every two years.”

- Means computers become faster, smaller, and cheaper over time.

📘 Example:
1980s CPUs → thousands of transistors
2020s CPUs → billions of transistors.

## 🔋 16. Power Consumption

### ⚙️ Dynamic Power

- Power used when transistors switch ON and OFF.

- Formula:
Power = ½ × Capacitance × Voltage² × Frequency

- Reduce power by lowering voltage or clock frequency.

### 💤 Static Power

- Power leakage even when circuit is idle.

- Becomes significant as transistors shrink.

### ⚙️ Techniques to Save Power:

- Sleep modes

- Clock gating (turn off unused parts)

- Lower voltage/frequency when idle

- Multiple low-frequency cores (instead of one high-speed core)

## ⏱️ 17. Computer Performance Metrics

Performance is measured by how fast a CPU runs a program.

### 🔹 Key Terms

- IC = Instruction Count (how many instructions executed)

- CPI = Cycles Per Instruction

- CCT = Clock Cycle Time

📘 Formula:

CPU Time = IC × CPI × CCT

Or,

Performance = 1 / Execution Time

📘 MIPS (Millions of Instructions Per Second):

MIPS = Clock Rate / (CPI × 10⁶)

## 📊 18. Amdahl’s Law

- Describes limits of speedup from improvements.

- If only part of a program is improved, total speedup is limited.

📘 Formula:

Speedup = 1 / [(1 – f) + (f / S)]

Where:

- f = fraction improved

- S = speedup of improved part.

📘 Example:
If 40% of a program can be made 5× faster:
Speedup = 1 / [(1 – 0.4) + (0.4/5)] = 1.56× overall.

## 🧭 19. Performance Trends

- Since 2004, chipmakers (Intel, IBM, AMD) shifted from increasing clock speed to multi-core designs.

- Power and heat limited frequency scaling.

- So now, CPUs use parallelism — multi-core, multi-threading, and vector units — to boost speed.

## 🔚 20. Summary

- Computer Architecture → What the computer does.

- Computer Organization → How it does it.

- ISA defines the boundary between software and hardware.

- Parallelism improves performance.

- Moore’s Law predicts continuous hardware improvement.

- Power and heat are now main limits of speed.

- Amdahl’s Law shows that improvement of only one part gives limited benefit.

- Modern processors rely on multi-core and RISC architectures for performance.

Would you like me to:

- 💾 Make this into a clean, colorful PDF with tables and diagrams, or

- ✍️ Continue with Lecture 3 SAP-1 in the same style (combined as one full chapter)?